Page 1 of 4

Procter & Gamble CPG TN6

2 Procter & Gamble Plaza Cincinnati, OH 45202

MATERIAL SAFETY DATA SHEET

Issue Date: 8/01 Issue Date: N/A

Supersedes: 3/00

SECTTONI - CHEmCALPRQDUCT Identity: Disinfecting Liquid Hard Surface Cleaner Brands: SPIC & SPAN Disinfecting Ail-Purpose Spray & Glass Cleaner Ready to Use (Professional Line)

Hazard Rating: 2	Health:   1	4=EXTREME

Flammability: 2	3=HGH

Reactivity: 0	2= MODERATE

1=SLIGHT

Emergency Telephone Number -1-800-332-7787 or call Local Poison Control Center

•

| fe:;.:;; . ..:;',;•:!';.;"•:,. • .-, .,VW,:.- SECTION H-COMPOSITION AND INGREDIENTS., : .;.'. iV^.:. -. V,;;;; -.. r Ingredients/Chemical Name: Cleaning agents (ethanol, butoxypropanol), quaternary, water, colorant and perfume. Hazardous Ingredients as defined by OSHA, 29 CFR 1910.1200. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Chemical Name | Common Name | CAS_Na | Recommended Limits | Composition Range | LD50/LC50 |
| Ethyl alcohol | Ethanol | 64-17-5 | ACGIH TLV: 1000 mg/m3 | 1-4% |  |
| 2-aminoethanol | Monoethanol-amine | 141-43-5 | ACGIH TLV: 3 ppm OSHA PEL: 3 mg/m3 OSHA PEL: 6ppm(15min) | 0.1-1% |  |

SECnONin - HAZARDS IDENTIEICAIION

Health Hazards (Acute and Chronic):

Ingestion:       May cause mucous membrane irritation. Possible mild gastrointestinal irritation with

nausea, vomiting and/or diarrhea. Large ingestions may cause temporary dizziness, uncoordination and headache.

Eye Contact:      May cause eye irritation. May result in transient, superficial effects similar to those produced by mild toilet soaps.

Skin:	May cause skin irritation. Use on irritated or extremely dry skin may aggravate the existing

condition.

Inhalation:         May cause mucous membrane irritation.

.-.-,.".: :	:	V         . '

- '   -."         "	-

Emergency and First Aid Procedures:

Ingestion:	Dilute with fluids and treat symptomatically. For large ingestions, call a physician.

Eye contact:        Flush thoroughly with water for 15 minutes. Get medical attention if irritation persists.
Skin:	Rinse exposed area with soap and water and discontinue use.

Inhalation:	Get fresh air.

Other: Product package has a caution statement: KEEP OUT OF REACH OF CHILDREN. CAUTION: Causes eye
irritation. Avoid contact with eyes, skin or clothing. Wash thoroughly with soap and water after handling.

Spic & Span Disinfecting Ail-Purpose Spray & Glass Cleaner RTU MSDS (Continued)

Page 2 of4

SECTION V - FIRE FIGHTING INFORMATION

Flash Point (Method Used): 136riF (cc)	Explosive Limits. LEL: N/A	UEL: N/A

Extinguishing Media: CO2, dry chemical, water, or "alcohol" foam.

Special Fire Fighting Procedures: Use water to keep fire exposed containers cool.

Unusual Fire Hazards: Combustible liquid. Product will produce a brief flash but will not support sustained combustion when in contact with open flame above the indicated flashpoint.

SECTION VI - ACCIDENTAL RELEASE MEASURES

Personal Precautions: N/A

Environmental Precautions: DISPOSAL IS TO BE PERFORMED IN COMPLIANCE WITH ALL FEDERAL, STATE AND LOCAL REGULATIONS. Small quantities may be slowly flushed down sewer with excess water or dispose as liquid scrap. Discard empty container in trash.

Steps To Be Taken in Case Material is Released or Spilled: Use water spray or dilute and/or wash away spills to avoid exposure and to protect persons working to stop/repair leak.

;         .	'."•     '. V: - v-.--•

Precautions To Be Taken in Handling and Storing:. Handling and storage must be well ventilated, cool and dry. Materials must be isolated from potential sources of ignition. Fire fighting equipment must be available at all times.

Other Precautions: Keep away from sparks and open flame.

SECTION Vm - EXPOSURE CONTROLS, PERSONAL PROTECTION

Respiratory Protection (Specify Type): None required with normal use.
Ventilation	General

Eye Protection: None required with normal use.

Industrial Setting: If splash of solution is likely, chemical goggles may be needed.

Protective Gloves: None required with normal use.

Industrial Setting; Minimize skin contact with protective gloves (rubber, neoprene).

Other Protective Equipment: None required with normal use.

Industrial Setting: Use ventilation to minimize exposure to vapor or mist (ethanol).

.SECTION IX - PHYSICAL, A^D GHEIWC^.PROPERTIES ? ^tV:   •

Boiling Point °F:2007f

Vapor Pressure (mm Hg): N/A

Vapor Density (Air=l): N/A

Odor Threshold: N/A

Coefficient of Water/Oil Distribution: N/A

Scooped Density: N/A

Appearance and Odor: Light blue liquid, solvent odor

Specific Gravity (H2O=1): 0.99 Percent Volatile by Volume (%) ca. 95 Evaporation Rate (nBuOAc=l): N/K Freezing Point: N/A pH (100% solution): 10.7 Solubility in Water:  Completely Reserve Alkalinity: N/A

Spic & Span Disinfecting Ail-Purpose Spray & Glass Cleaner

Spic & Span Disinfecting AH-Purpose Spray & Glass Cleaner RTU MSDS (Continued)	Page 4 of 4

SECTION XVI -

*N/A. - Not Applicable

*N/K_- Not Known

The submission of this MSDS may be required by law, but this is not an assertion that the substance is hazardous when used in accordance with proper safety practices and normal handling procedures. Data supplied is for use only in connection with occupational safety and health.

The information contained herein has been compiled from sources considered by Procter & Gamble to be
dependable and is accurate to the best of the Company's knowledge. The information relates to the specific material
designated herein, and does not relate to the use in combination with any other material or any other process.
Procter & Gamble assumed no responsibility for injury to the recipient or third persons, for any damage to any
property resulting from misuse of the controlled product